# Power-System Rebuild and Hardware Reverse-Engineering of a Spot Hip Actuator

CDE2605 Undergraduate Research Opportunities Programme (UROP) · AY2025/2026 Special Term

Yizhang [full name — TBC] · [Matriculation No. — TBC]

AY2025/2026 Special Term

Table of Contents

List of Figures	3

List of Tables	4

Introduction	5

Background	6

Battery module architecture	6

Hip-X actuator architecture	7

Methodology 1 — Power-system rebuild	7

Fault diagnosis and the decision to rebuild	7

Teardown and OEM construction	9

Cell sourcing	11

Pack design	12

Assembly and spot-welding	13

Testing, fault-finding and rework	15

Second module: a V2 pack that validated the fixes	18

Methodology 2 — Actuator mechanical investigation	19

Teardown and driver comparison	20

Driver and cable swaps	22

Motor disassembly — discovering the secondary encoder	22

The output-encoder swap	24

Methodology 3 — Encoder electrical reverse-engineering	25

Board components and pinout	26

Configuration-EEPROM decode	27

Signal routing	29

Bench read attempt and its limit	29

The DB25 payload-port bypass	31

Evaluation, Limitations and Improvements	31

Power system	31

Actuator, at the hardware level	32

Conclusion	33

References	33

Appendices	34

Appendix A — Project timeline (Gantt)	34

Appendix B — iC-MU EEPROM configuration register map	35

Appendix C — Encoder-board pinout	37

Appendix D — DB25 payload-port cover pin shorts	38

Appendix E — Battery CAD iterations	38

# Introduction

The Boston Dynamics Spot quadruped held by the programme was received non-operational, and this report documents the hardware-level work carried out to return it toward service. Two independent faults were present on arrival. First, the power subsystem was dead: both of Spot’s battery modules refused to charge. Second, once external power was restored, one leg actuator, the left hind hip-X (abduction/adduction) joint (hereafter hip-X, and specifically left_hind_x), behaved abnormally, with the joint failing to reach commanded positions while the other three legs moved normally. The mechanical and electrical nature of that actuator was, at the outset, entirely unknown: it had never been opened, and no internal documentation was available.

The first objective was to restore the power subsystem by diagnosing the charge fault and, should the cells prove unrecoverable, rebuilding each pack on its original battery management system (BMS) in the OEM form factor. The second objective was to narrow down fault down to a specific component through a sequence of component swaps, then characterizing the actuator as a piece of hardware, covering its mechanical construction and the electrical design.

The scope of this report is deliberately confined to the hardware layer. It describes physical acts (teardown, cell replacement, spot-welding, component interchange, board probing) and the hardware and electrical facts observed during them. The complementary diagnostic interpretation of the actuator’s control-signal behaviour, together with the eventual encoder repair, was carried out on the signal/software layer and is reported in the companion report [Ming, SS-TBD]; those results are cited here as single premise sentences.

The working approach throughout was bench-based and comparative. Because Spot’s two hind legs are nominally identical, the healthy hind leg served as a reference against which the faulty one could be compared part by part, a strategy that recurs across the actuator chapters.

# Background

## Battery module architecture

Spot is powered by two identical, hot-swappable battery modules. Each module is a sealed assembly whose perimeter is held closed RTV compound. The internal architecture was first established from a published third-party teardown of the Spot platform [3] and then confirmed directly during the project’s own disassembly (Section 4). Each module carries 56 Samsung INR18650-30Q lithium-ion cells, a 3000 mAh cell rated 15 A continuous / 30 A peak, identified from Boston Dynamics’ Spot Battery Safety Data Sheet [1], arranged in a 14s4p configuration and physically built as two 7s4p sub-packs (labelled A and B). A single BMS PCB, built around an STM32F1xx microcontroller, manages the module and communicates with the robot over a CAN bus. Table 1 summarises the construction as found.

Table 1 — OEM battery module architecture, as established from the teardown report [3] and confirmed on disassembly.

| Property | Value |
| --- | --- |
| Nominal energy | ~500 Wh |
| Rated runtime | ~1.5 h |
| Cell | Samsung INR18650-30Q (3000 mAh, 15 A cont. / 30 A peak) |
| Cell count & topology | 56 cells in 14s4p, built as two 7s4p sub-packs (A & B) |
| BMS | Single PCB, STM32F1 series MCU, CAN bus to the robot |
| Measurement pads | 28 pads: positives along the long PCB edges, common negative down the centre |
| Thermal sensing | One flexible PCB per sub-pack, each carrying five SMD NTC thermistors |
| Thermal / safety | Phase-change material (PCM) around the cells; fish-paper terminal insulation; foam seating |

Two construction details established here proved decisive for the rebuild. The cell layout is deliberately irregular: cells are unevenly spaced and are not even collinear within a row, the centre cells sitting slightly proud of their neighbours, so any replacement spacer had to reproduce those exact positions rather than assume a regular lattice. Thermal management is provided by a phase-change material packed around the cells, which absorbs heat through its latent heat of fusion and buffers thermal-runaway energy, supplemented by the two five-element NTC-thermistor strips that monitor temperature across each sub-pack.

## Hip-X actuator architecture

Each of Spot’s legs is driven by three actuators; the one of interest here is the hip-X actuator, which abducts and adducts the leg. Its mechanical output is produced by a brushless motor turning through a 50:1 harmonic drive (strain-wave reducer), so that a large number of motor revolutions map to a small, high-torque motion of the leg. Bench measurements on the hip motor established a 20-pole, 18-slot machine (a slot/pole ratio of 0.9) with a phase resistance of approximately 0.5 Ω.

The actuator carries two angle sensors rather than one, a fact that only became apparent once the motor was opened (Section 6). The primary encoder is a motor-side iC-MHM 14-bit absolute Hall encoder that reads rotor angle before the reduction. The secondary output encoder is a separate iC-Haus iC-MU Hall-IC mounted vertically beside the harmonic-drive output stage, reading the actual joint angle after the 50:1 reduction using a 32-pole radial Nonius magnetic encoder ring. The secondary encoder sits on its own small PCB, which also carries an external configuration EEPROM and an RS-485/RS-422 differential transceiver, stacked with the joint’s load-cell PCB behind the main motor connector. The physical construction and electrical routing of that board are characterised in this report (Sections 6 and 7); the functional role of the two encoders within Spot’s joint-control signalling is described in [Ming, SS-TBD].

# Methodology 1 — Power-system rebuild

## Fault diagnosis and the decision to rebuild

The power subsystem was the first blocker: with both modules unable to charge, Spot could not be run from its own batteries. On the OEM charger the charge indicator flashed red briefly and then went dark, rather than the steady green blink that signals normal charging. The handheld controller, by contrast, powered on and connected normally (though it drained quickly), confirming that the fault was specific to the battery modules rather than the wider system.

To gain direct access to the cells and the BMS measurement pads, a module was opened along its adhesive seam. The initial cell-group voltages averaged only about 90 mV, far below the 2.5 V minimum safe voltage for an 18650 cell, indicating severe over-discharge. A staged recovery was nonetheless attempted before any decision to scrap the cells: each accessible group was bench-charged slowly at 2.0 V / 1.5 A across the BMS tabs, and once groups reached roughly 2.0 V the limits were raised to 16 V / 5 A across up to five groups at a time (Figure 2). The pack rose to about 14.5 V on the supply (P1-GND measured 13.3 V combined), demonstrating that it would take charge.

Figure 1 — Connecting a module to the OEM charger. The indicator flashes red and dies rather than blinking steady green, signalling a refused charge.

Figure 2 — Staged bench-charging of the individual cell groups, an attempt to nurse them back above the safe minimum before committing to a rebuild.

The recovery did not hold, and the manner of its failure was itself the diagnosis. Returned to the OEM charger, the BMS status LED blinked, a successful CAN handshake, but the charge indicator went green and then static, reporting a “full” charge on cells that were plainly empty (Figure 3). The Spot manual attributes a premature “full” to highly imbalanced cells and suggests leaving the pack plugged in to auto-rebalance, which had no effect. After idling, the cells were warm to the touch and pack voltage collapsed from about 1.6 V to 0.6 V. The combination of warmth and rapid voltage collapse is consistent with resistive self-discharge through dead cells now behaving as internal short circuits; the cells were therefore judged degraded beyond recovery. The decision followed directly: replace the cells rather than recover them and, to preserve the OEM BMS, CAN interface and casing, rebuild onto the original board in the standard 14s4p configuration by spot-welding replacement cells onto the original pads.

Figure 3 — The OEM charge indicator sitting static green (“full”) moments after the pack measured near-empty: the signature of dead, imbalanced cells.

## Teardown and OEM construction

Having resolved to rebuild, the module was fully disassembled, both to strip out the dead cells and to document the OEM construction so the replacement pack could match it (Figure 4). The power cables were freed by cutting away a large securing glue blob piece by piece, then unscrewing the leads and unplugging the CAN cable; a plastic insulator covering three of the BMS pad sets was removed. The nickel strips are spot-welded to the BMS pads and could not be cut cleanly because the plastic frame blocked the cutter, so they were pried off the pads, leaving small welded-nickel bumps that required clean-up before re-welding. The two 7s4p sub-packs were then separated: a long plastic snap-connector releases by prying both ends and twisting the centre clips, and a shorter connector by pushing its centre axle out from below. The two flexible thermistor strips (Figure 7) were unplugged and labelled A and B to preserve their original sides.

Figure 4 — The module fully separated into its sub-packs, BMS, frame, connectors and thermistor strips.

The teardown confirmed the topology of Table 1 and fixed the two design constraints noted in the Background: the irregular, non-collinear cell layout (Figure 5) that the replacement spacer would have to reproduce, and the 28-pad BMS layout (Figure 6), positives along the long edges, common negative down the centre.

Figure 5 — The OEM cell arrangement. Spacing is irregular and the centre cells in each row sit proud of their neighbours.

Figure 6 — The BMS PCB with cabling removed, showing the pad layout onto which the replacement packs weld.

Figure 7 — One of the two flexible PCBs, each carrying five SMD NTC thermistors thermally bonded to the cells.

## Cell sourcing

The replacement cell was fixed by the OEM specification: the Samsung INR18650-30Q identified from the Spot Battery Safety Data Sheet [1], of which 56 were required per module. Because local suppliers did not stock the exact cell, several options were compared (Table 2). The original 30Q was selected to keep the rebuild electrically identical to stock, the cheapest source being Falcon PEV [2]; higher-capacity alternatives were rejected as unnecessary deviations from the OEM design.

Table 2 — Replacement-cell sourcing comparison. The OEM cell was chosen to keep the pack identical to stock.

| Cell | Capacity | Discharge | Notes |
| --- | --- | --- | --- |
| Samsung INR18650-30Q (chosen) | 3000 mAh | 15 A / 30 A peak | OEM spec; cheapest source found (Falcon PEV) [2]. |
| LG HG2 | 3000 mAh | 20 A | Same capacity, slightly higher rating (~S$663 for 56). |
| Samsung INR18650-35E | 3500 mAh | higher | More capacity and current, but pricier. |

## Pack design

Two design questions had to be settled before assembly: how to size the inter-cell conductors, and how to hold the irregular cell lattice rigidly.

The conductor sizing followed from an estimate of Spot’s power draw. With the pack rated at about 500 Wh and a datasheet runtime of roughly 1.5 h, and assuming discharge to 20 % state of charge over that time, the average power is approximately 0.8 × 500 / 1.5 ≈ 267 W; against the manual’s 38-52 V supply this implies an operating current of about 5.1-7.0 A. Sizing to a 50 % margin (~400 W continuous) gives a peak pack current of about 10.5 A, or roughly 2.63 A per cell across the four parallel cells in each group. Those figures set the strip widths in Table 3; the OEM pack uses 12 mm strip throughout, which is comfortably conservative for the actual current.

Table 3 — Inter-cell conductor sizing, from the estimated ~10.5 A peak pack current.

| Conductor | Spec | Basis |
| --- | --- | --- |
| Inter-cell links | 0.1 × 8 mm pure nickel | Per-cell current only ~2.63 A |
| 4s pack leads | 0.15 × 12 mm pure nickel | Rated ~17 A optimal / 25.5 A acceptable, ample for ~10.5 A pack max |

The irregular OEM layout was replicated in CAD from direct measurement of the original pack and 3D-printed as a four-piece cell spacer in PC-CF (polycarbonate / carbon-fibre), chosen for its high operating-temperature limit and dimensional stability for rigidity (Figure 8). The four spacers printed over roughly ten hours. Safety and thermal provisions mirrored the OEM approach where practical: fish-paper insulation cut to shape across the cell terminals to prevent accidental shorts, and RTV compound between the pack and the shell to seat the cells firmly in the casing grooves. An off-the-shelf phase-change material equivalent to the OEM latent-heat filler [4] was researched but no solutions easy to work with was found, so it was omitted from this rebuild, on the basis that the current-sizing estimate above already showed the cells operating well under their rated capacity even at peak load: a deviation from stock noted here as a limitation.

Figure 8 — The four 3D-printed PC-CF cell spacers, which replicate the irregular OEM cell positions.

## Assembly and spot-welding

The cells were seated in the spacers and their terminals joined with nickel strip at Sodion Energy. Only 10×0.15 mm strip was available, so it was trimmed to about 8 mm before use (Figure 9). The 12 mm BMS PCB tabs were thicker and tended to pop off since they were stacked on up to 2 layers of 8mm strips, hence some were soldered rather than welded. A recurring practical constraint was that the welder could not reliably join more than one nickel strip at a time, which ruled out strip-stacking and shaped the later V2 design.

Figure 9 — A cell sub-pack with nickel strips spot-welded across the terminals.

Welder reliability was in fact the dominant obstacle of the whole rebuild. Partway through, the bench spot-welder failed with an Error 22 (transistor fault): after about thirty minutes of use there was an audible pop and a smell of smoke from the supply. The immediate contributor was assessed as thermal saturation of the welder’s IGBTs; the underlying cause was later traced, on opening the unit, to a cracked solder joint on a capacitor module, with burn marks on the PCB beside an otherwise-healthy capacitor. A handheld spot-welder (Figure 10) was pressed into service to finish both packs while the bench unit was out of action.

Figure 10 — The handheld spot-welder that completed the job after the bench welder failed with an Error 22 transistor fault.

The pack was then reattached to the BMS, ground ends first and positive ends second, in no specific order. As each cell group was welded on, the BMS’s red indicator LEDs blinked in response, but in no consistent order: static for a few groups, blinking for another few, unlit for the rest (Figure 11). The module was finally closed with its original foam pieces reinserted to keep the cells seated.

Figure 11 — BMS indicator LEDs responding as each cell group is welded onto the board.

## Testing, fault-finding and rework

The first charge test exposed the rebuild’s central weakness. The state-of-charge (SoC) button showed a single blinking green bar indicating a low initial charge; the charger flashed green (charging) but, after about twenty minutes, went static green (“full”) while the SoC only showed one bar. The BMS then flashed three red LEDs and extinguished its green STM32 LED, halting the charge.

From the admin console the battery balance index read 0.213, well above the 0.1 threshold. Boston Dynamics suggests leaving the pack to auto-balance or connecting the pack to the charger overnight for active cell rebalancing. The latter, however, requires battery firmware later than V45, whereas the pack reported V33 in the admin console.

The battery was left disconnected overnight in an attempt to passively balance the cells. A charge test next day, however, revealed the same issue of the battery refusing to charge beyond a single SoC bar.

The rebuilt pack did nonetheless power the robot through a boot cycle, confirming that it could deliver load current even while imbalanced. That ruled out a dead pack outright and pointed the investigation at the cells’ connections rather than at the charging system.

After charging until the charger again reported “full”, each group’s voltage was measured and tabulated (Figure 12). Groups 1, 2 and 8 read abnormally while the remainder were well balanced, and inspection tied every anomaly to a bad weld on the positive side: on groups 2 and 8 the positive weld had popped off, leaving only two of the four parallel cells connected; on group 1 a defective weld loosely joined the nickel strips, producing a high contact resistance. A resistance comparison across sound and poor joints (Figure 14) confirmed that an inconsistent weld raises the trace resistance substantially.

Assuming the BMS provides a constant current to each cell group, the groups with defective welds will charge faster since there are effectively less batteries connected in parallel. This causes the cell group’s voltage to rise much faster, resulting in the heavy imbalance observed.

Figure 12 — Per-group voltage sweep after the first full charge; groups 1, 2 and 8 read abnormally relative to the balanced remainder.

Figure 13 — A defective / popped weld joint of the type responsible for the group imbalance.

Figure 14 — Resistance across a sound weld (left) versus a poor one (right). The poor joint reads a substantially higher resistance.

The weld problem then escalated from imbalance to a hard lockout. After the pack was left overnight, Spot reported a battery fault and refused to power on, and charging produced the same fault with all SoC lights blinking. A third voltage sweep (Figure 15) showed group 1 reading an impossible −0.5 V while every other group, including the previously unbalanced ones, sat at a healthy 3.71-3.72 V. Candidate failure modes were narrowed by elimination: no visible BMS damage argued against a blown MOSFET or resistor; normal resistance along the positive nickel strips ruled out a positive-side weld. The group only read 3.71 V  when the multimeter probe was reached down and pressed directly onto a cell’s negative terminal (Figure 16). That intermittent contact isolated the fault to a broken weld on the group’s negative terminal: the group had effectively disconnected from the BMS, and the −0.5 V was a phantom reading, most plausibly the reverse bias of an internal protection diode or the drop across a balancing bleed-resistor on the BMS.

Figure 15 — Third voltage sweep. Every group is balanced at ~3.71 V except group 1, which reads a negative voltage.

Figure 16 — Group 1 reads a phantom −0.5 V at the BMS pads (left) while probing the cell group directly reads a healthy 3.71 V (right), confirming a broken negative-terminal weld, not a bad cell.

Rather than repair only the known-bad joints, the pack was brought back to Sodion Energy to have all the BMS  welds undone and the packs separated for a detailed check to eliminate any further intermittent connections. After re-welding, the SoC fault lights cleared, the pack charged normally, and it reached a full, balanced charge for the first time (Table 4). Battery module #1 was thereby repaired and verified.

Table 4 — Battery pack #1 state before and after the weld rework.

|  | Before rework | After rework |
| --- | --- | --- |
| Group 1 (P1-P2) | −0.5 V (disconnected) | balanced, ~3.7 V |
| Charge behaviour | premature false “full”, fault lockout | charges to full, no fault |
| Cell balance | groups 1 / 2 / 8 anomalous | all groups balanced |

## Second module: a V2 pack that validated the fixes

The single dominant lesson of pack #1, that reliability was governed almost entirely by spot-weld quality and that weld failures traced to bends and stress points in the nickel-strip routing imposed by the bracket design, was carried directly into a second module.

The V2 brackets (Figure 18) added a further set of braces with tighter tolerances (white) to hold the cell lattice rigidly (pack #1’s brackets had allowed some play when flexed), reincorporated the OEM injection-moulded spacer positions for rigidity, and were re-dimensioned to fit the casing and PCB more accurately after the lessons of V1. They were printed in glass-filled ABS (ABS-GF) for dimensional stability at temperature, and the BMS mounting was changed from expanding rivets, which had split the printed layer lines, to captured M3 nuts under 1.2 mm of plastic, secured with M3×6 mm screws.

A set of custom 0.15mm thick nickel tabs were also designed around the spacers to eliminate the nickel strip trimming and stacked-welding process in V1, which was tedious and time consuming.

One design feature of the custom nickel tabs worth noting is the 0.8mm width slot cutout at every weld site. The spot welder probes are positioned across the slot when welding. This geometrical feature encourages the spot welder’s current to flow down through the nickel layers instead of just shorting across the surface, producing much stronger and hence reliable welds with little to no sparking.

Figure 17: Slot cutout encourages an inter-layer current path rather than shorting across the surface.

Figure 18 — The pack #2 V2 bracket CAD, adding braces (white), OEM spacer positions and custom nickel tabs.

The improvement was borne out in assembly (Figure 19). After assembly, the SoC button was initially unresponsive; this was traced to a sheared SoC flat-flex cable near its base (Figure 17). The broken cable was repaired by scraping back the insulation on both ends of the flex PCB and soldering the exposed traces, with kapton tape added as a strain relief on the joints. The SoC then read correctly. Pack #2 charged to full within about an hour, reported a very healthy balance index of 0.02, and accepted a battery firmware update once installed in Spot. Both modules were thereby restored to a full, balanced charge, meeting the power-system objective.

Where pack #1 took two people three days and had multiple points of failure after assembly, pack #2 was completed by one person in a single afternoon and verified to have no faults, testifying the effectiveness of the design upgrades.

Figure 17 — The sheared SoC flat-flex cable (left) and the soldered, kapton-secured repair (right).

Figure 19 — Pack #2’s laser-cut nickel strips (left) and the completed BMS weld (right), both markedly cleaner than pack #1.

# Methodology 2 — Actuator mechanical investigation

With power restored, attention turned to the faulty left_hind_x actuator. The investigation proceeded as a sequence of physical component swaps, each designed to move one candidate part between the faulty left and the healthy right hind hip-X actuators and observe whether the abnormal behavior physically followed the part or stayed with the socket. This chapter reports the swap procedures and the physical observations; for each swap, the corresponding diagnostic reading and its interpretation belong to the signal layer and are reported in [Ming, SS-TBD].

## Teardown and driver comparison

Spot was disassembled to extract the left hind leg together with its hip-X motor and controller (Figure 20). During teardown the leg wiring and controller board were inspected for physical defects and none were found; a layer of what appeared to be epoxy coating over the controller board frustrated any attempt to probe its test points, as contact could not be made without forcibly scraping the coating away. Because a purely visual inspection could not distinguish a mechanically sound motor from a magnetically faulty one, the investigation was framed around comparison with the healthy right hind leg.

Figure 20 — Spot opened on the bench to remove the left hind leg and its hip-X actuator.

Both hind legs were then removed and their hip-X motor driver boards compared side by side (Figures 21 and 22). The two drivers were physically identical, with no sign of component degradation on either. Rotating each motor by hand confirmed that the encoder’s diametric rotor magnet was firmly seated, rotated freely, and stood at an identical protrusion height on both sides, ruling out an obviously displaced or loose magnet as the cause.

Figure 21 — The two hind hip-X motor driver boards removed for direct comparison.

Figure 22 — The driver board, encoder side, showing the motor-side primary encoder region.

## Driver and cable swaps

Two swaps tested the driver electronics and the wiring. First, the left and right hip-X driver boards were exchanged and the legs refitted; on power-up no motor faults were reported, but the abnormal joint behavior physically remained with the left socket rather than following the swapped board. Second, the hip-X motor cables were exchanged, after probing the connector PCBs to confirm they were not mirror images and that power and ground would map correctly, and again the abnormal behaviour stayed with the left socket while the other joint operated normally. Table 5 records the physical actions and observations; in both cases the diagnostic reading that accompanied the swap, and the inference drawn from it, are reported in [Ming, SS-TBD].

Table 5 — Actuator component-swap actions and the physical observations recorded. The diagnostic reading and interpretation for each swap are reported in [Ming, SS-TBD].

| Physical action | Physical observation |
| --- | --- |
| Exchange L/R hip-X driver boards | No motor faults on power-up; abnormal behaviour stayed with the left socket, not the board. |
| Exchange L/R hip-X connector cables | Abnormal behaviour stayed with the left socket; the other joint responsive and normal. |
| Exchange entire rotor + output-encoder + load-cell assemblies | Behaviour moved to the right socket, but a load-cell hard fault appeared (load cells are calibrated per joint). |
| Exchange output-encoder PCB only (rotors/load-cells returned to original sockets) | Abnormal frozen behaviour cleared on both joints; a residual mechanical offset remained on one joint. |

Taken together at the hardware level, the driver and cable swaps physically eliminated the driver electronics and the motor wiring as the location of the fault, since neither swap moved the abnormal behaviour with the part. That left the angle-sensing hardware inside the motor as the remaining candidate, which motivated opening the motor itself.

## Motor disassembly — discovering the secondary encoder

Each hip-X motor was disassembled to further identify potential points of failure. To isolate the hip-X (abduction) motor, the hip-Y (flexion) motor was first removed from the rotor housing by undoing four very tight M5 Torx screws, after which a flange serving as both cover and mechanical end-stop was freed by removing eight torqued-down M3 hex screws; the X-motor assembly, held together without the flange only by a snug fit between the rotor bearing and housing, could then be pulled apart with some force. The hex screws used by Boston Dynamics were noted to be relatively soft, several stripping easily during disassembly. A further construction detail was that the internal screws on the aluminium plate holding the stator in place are numerically labelled, suggesting a defined tightening sequence to keep the rotor centred.

The disassembly revealed the key architectural fact of the whole investigation: a second Hall sensor, also from iC-Haus (the iC-MU), mounted beside the harmonic-drive stage (Figures 23 and 24). This was the secondary output encoder introduced in the Background, not visible in any earlier inspection. Because the driver and cable swaps had already cleared the electronics and wiring, and because the primary iC-MHM rotor encoder is a robust four-Hall device whose reading is insensitive to small magnet-placement deviations (the rotor magnet having already been shown well-seated on both sides), this newly exposed secondary encoder became the primary hardware suspect for the abnormal joint behaviour. The sensor tracks a 32-pole Nonius magnet ring mounted on the output of the harmonic drive, indicating that it is this encoder, not the motor-side iC-MHM used to commutate the BLDC drive pre-reduction, that is responsible for the actual joint-angle read-out.

Figure 23 — The secondary output-encoder Hall-IC (iC-Haus iC-MU) found at the harmonic-drive output stage of the hip-X motor.

Figure 24 — The 50:1 harmonic drive of the hip-X actuator.

## The output-encoder swap

The decisive swap concerned the output encoder itself. As a first step, the entire rotor assembly, main connector, encoder and force-feedback (load-cell) module together, was exchanged between the two hind hip-X sockets. This moved the abnormal behaviour to the right socket as intended, but it also raised a load-cell hard fault: the load cells are calibrated per joint, so swapping them was not admissible, and the fault prevented the motors from powering on. The swap was therefore refined (Figure 25): only the main connector PCB carrying the secondary output encoder was exchanged, while each rotor and its load cell were returned to their original stator. The driver boards were likewise returned to their original sides.

Figure 25 — The rotor / output-encoder assemblies during the swap; only the secondary output-encoder PCB was ultimately interchanged, with the load cells kept in their original joints.

With the output-encoder PCBs interchanged between the hind hip-X sockets and the magnets held fixed, the load-cell fault disappeared and both joints thereafter responded to articulation: the previously frozen behaviour was gone (Figure 26). A residual mechanical offset remained on the left joint, measurable as a consistent difference in the joint’s centre position relative to the opposite side when each leg was jogged by hand across its full range of motion; the per-joint offsets recorded in this way are given in Table 7. The physical range (the delta between the mechanical limits) was consistent across all four legs, indicating that the offset was a fixed calibration/seating shift rather than a change in travel.

Figure 26 — The secondary output-encoder PCB, close up, after the swap.

Table 7 — Hind hip-X mechanical offsets from the horizontal centre, measured by manually jogging each leg to its limits. The physical range was consistent across joints. Interpretation of the readings is reported in [Ming, SS-TBD].

| Joint | Offset from centre |
| --- | --- |
| Right hind hip-X (reference side) | ~0.12 rad (≈6.8°) |
| Left hind hip-X (faulty side) | ~0.53 rad (≈30°) |

At the hardware level the swap sequence was conclusive in one specific sense: interchanging the secondary output-encoder PCB physically moved the abnormal behaviour between joints, whereas interchanging the drivers, cables, rotors and magnets did not. The fault therefore resides in a physical hardware difference at the secondary output encoder, and not in the driver, wiring, rotor or diametric magnet. The electrical nature of that board, and the question of what, precisely, could differ between two nominally identical encoders, is taken up in Section 7.

# Methodology 3 — Encoder electrical reverse-engineering

Localising the fault to the secondary output-encoder board raised a hardware question that could only be answered by reverse-engineering the board’s electrical design: what does the board consist of, how is it wired, and could a configuration or connection difference, rather than a dead component, explain the behaviour? This chapter reports that reverse-engineering: the board’s components and pinout, the decoding of its configuration EEPROM, the routing that connects it to the rest of the actuator, and the bench attempt to read the sensor directly.

## Board components and pinout

The secondary output-encoder PCB carries three principal devices: the iC-MU Hall-IC itself, an external EEPROM holding its configuration, and an RS-485/RS-422 differential transceiver (an Analog Devices LTC2863 [6]) that carries the sensor’s serial output off-board (Figure 27). A set of test pads exposes the EEPROM’s SDA/SCL lines, presumably for programming, together with the transceiver’s differential outputs and supply near the main connector.

Figure 27 — The secondary output-encoder PCB, showing the iC-MU Hall-IC, the external configuration EEPROM and the RS-485/RS-422 transceiver.

Probing established the board’s wiring as a textbook ExtSSI slave node (Figure 28, Table 6). Pin PA0 of the iC-MU is tied to ground, which selects the BiSS fall-back protocol; the transceiver’s receiver output (RO) drives PA1 (the clock input, MA), and the transceiver’s driver input (DI) is fed from PA3 (the slave data output, SLO); PA2 (slave in, SLI) is grounded, correct for a single unchained slave. This is exactly the connection the iC-MU datasheet [5] prescribes for its MODEA=7 ExtSSI mode, and it is consistent between the boards inspected.

Figure 28 — The iC-MU pinout and connections, wired as an ExtSSI slave node.

Table 6 — iC-MU Port-A (ExtSSI, MODEA=7) pinout, as wired on the board and cross-checked against the datasheet [5].

| iC-MU pin | Function (MODEA=7) | Connection |
| --- | --- | --- |
| PA0 | NPRES | tied to GND (selects BiSS fall-back) |
| PA1 | MA (clock in) | ← LTC2863 RO |
| PA2 | SLI (slave in) | GND (correct for a single, unchained slave) |
| PA3 | SLO (data out) | → LTC2863 DI |

## Configuration-EEPROM decode

To characterise the encoder’s configuration, the contents of its external EEPROM were decoded against the iC-MU datasheet [5], register by register. Per the datasheet, the iC-MU attempts to load its configuration from this EEPROM three times at start-up and, failing that, boots on default values, with the fall-back output protocol selected by PA0 (LOW → BiSS). The decode established that the stored configuration is a coherent, datasheet-normal ExtSSI set-up rather than anything corrupt or exotic: amplitude control is enabled (so the manual gain fields are intentionally inert), Port A is configured as ExtSSI and Port B as ABZ, the interpolation filter is set to the 14-bit / 39 dB FILT4 setting, the master/nonius period count MPC=5 gives up to 19-bit absolute resolution, and the config-range CRC16 (0x52A8) matches the value computed over the configuration bytes. Table 8 lists the most diagnostic registers; the full annotated map is given in Appendix B. The upper offset/preset region reads as erased (all 0xFF) with a non-matching CRC8, consistent with an unused absolute-offset block rather than a fault.

Table 8 — Selected iC-MU EEPROM configuration registers (full annotated map in Appendix B).

| Addr | Raw | Field(s) | Decoded meaning |
| --- | --- | --- | --- |
| 0x05 | 88 | ENAC=1, CIBM=8 | Amplitude control active; bias-current trim nominal (manual gains ignored). |
| 0x0B | 07 | MODEA=7, MODEB=0 | Port A = ExtSSI (NPRES/MA/SLI/SLO); Port B = ABZ. |
| 0x0E | 14 | LIN=1, FILT=4 | Linear/radial scanning; FILT4 = 39 dB, 14-bit interpolation. |
| 0x0F | 05 | MPC=5 | 32 master / 31 nonius periods; ≤19-bit absolute resolution. |
| 0x13-0x14 | FF 0F | RESABZ=0x0FFF | ABZ resolution = (0x0FFF + 1) × 4 = 16 384 edges. |
| 0x21-0x22 | 52 A8 | CRC16 | Matches the CRC computed over the config ranges: configuration is intact. |
| 0x3E-0x3F | 69 43 | MFG_ID | 0x6943 = ASCII “iC” (iC-Haus). |

The significance of this decode is that it removes configuration corruption from the list of possible faults at the hardware level: the board’s programmed set-up is internally consistent and datasheet-normal. The complementary comparative result, the bench read that established the faulty and healthy encoders to be byte-identical in their stored configuration, and hence that the difference between them is physical rather than programmed, was performed on the signal layer and is reported in [Ming, SS-TBD]; this report owns the decode, not that comparative read.

## Signal routing

Probing to trace the sensor signals out to the main motor connector showed that no such route exists: none of the transceiver’s differential A/B/Z/Y outputs reach the main connector. Instead, the secondary output encoder is wired directly to the motor driver’s STM32 microcontroller, which consumes the post-reduction joint angle locally, formats feedback, and streams it to the main compute module over the driver’s own CAN transceiver. This routing is a hardware finding in its own right, and it also explains, at the hardware level, why the driver rather than the main computer governs the joint’s local behaviour; the control-loop consequences of that routing are a signal-layer matter and are discussed in [Ming, SS-TBD].

## Bench read attempt and its limit

A final effort was made to read the iC-MU directly on the bench while it remained installed in the actuator, so that the sensor’s raw output could be observed as hardware. The differential signal pairs proved inaccessible at the driver, but probing showed that the four test pads beside the main connector are the transceiver’s A/B/Y/Z differential pairs, with two further pads providing +5 V and ground (Figure 30). Wires were soldered to these pads (Figure 31) and the encoder section powered from a bench supply, drawing about 0.11 A at 5 V.

Figure 30 — The flex-PCB test-pad map: the four pads beside the main connector are the transceiver’s A/B/Y/Z differential pairs, with +5 V and ground alongside.

Figure 31 — Wires soldered to the flex-PCB test pads to power and monitor the encoder section on the bench.

The read did not succeed, and the reasons are themselves hardware findings. Establishing serial communication with the iC-MU over its RS-485/RS-422 transceiver required either a matching full-duplex four-wire transceiver or two standard RS-485 transceivers, neither of which was available. The alternative, driving the sensor with the motor driver’s own STM32 and observing the transceiver output lines on an oscilloscope (Figure 32), was blocked by a power issue: although the driver’s 5 V rail measured a correct 4.994 V, the 3.3 V rail that supplies the STM32 sat at only about 0.4 V. Tracing the supply showed that the 5 V feeds an LT3029 LDO regulator [7] whose 3.3 V output was the dead rail, and that the LDO appeared to be enabled by another rail rather than by the 5 V supply, so the microcontroller never powered. Powering the STM32 directly from the 3.3 V pad still produced no edges on the monitored A and Y lines, indicating that the driver was not, under these bench conditions, periodically polling the sensor to generate an output. The bench read therefore reached its limit: reading the sensor’s raw registers requires either the correct RS-485/RS-422 interface hardware or a way to command the driver’s STM32 to poll the encoder, and neither was available within the project’s timeframe. This dead end is carried forward as a limitation and a concrete piece of future work.

Figure 32 — The oscilloscope bench read attempt; no edges were observed on the monitored transceiver lines, the driver STM32 remaining unpowered on its 3.3 V rail.

## The DB25 payload-port bypass

One further electrical obstacle stood between the restored power system and any motor test, and it was resolved by reverse-engineering a connector. Enabling motor power raised an “uncovered payload port” stop function: Spot detects whether its DB25 payload-port covers are fitted, and one cover was missing. Inspection showed the cover contained no active components, only a mating DB25 shell, so the detection had to be passive. Probing the cover for continuity revealed that four sets of pins are shorted together inside it, and replicating those shorts on the open port, first with tinned jumper wires, later with a female DB25 header wired to match, cleared the lockout and allowed motor power to be enabled. The pin-short map is given in Appendix D.

# Evaluation, Limitations and Improvements

## Power system

Against its objective, the power-system work succeeded in full: both battery modules were rebuilt on their original BMS boards in the OEM 14s4p form factor and both reached a full, balanced charge, with pack #2 reporting an excellent balance index of 0.02. The single most important finding is that the rebuild’s reliability was governed almost entirely by spot-weld quality: the same class of defect manifested first as cell-group imbalance and then as a battery-fault lockout from one disconnected joint, and both traced back to bends and stress points in the nickel-strip routing imposed by the first bracket design. The V2 pack turned that finding into a design improvement: added bracing, OEM-derived spacer positions, laser-cut slotted strips that avoid stacking, and captured-nut BMS mounting. The result was a pack that welded cleanly and balanced correctly on the first attempt, assembled in a fraction of the time. Two limitations remain. The OEM phase-change thermal filler was not reproduced, because no workable off-the-shelf equivalent was found; the pack instead relies on the silicone thermal sheet and the retained NTC monitoring, and sourcing a suitable PCM is a clear improvement for a future build. And the pack #2 SoC repair depended on an improvised fastener (a cardboard shim in place of the missing flex-connector fastener), which should be replaced with a proper part.

## Actuator, at the hardware level

The mechanical investigation met its objective of localising the fault to a physical part. By moving each candidate part in turn between the faulty and healthy hind hip-X actuators, the hardware work physically ruled out the driver electronics, the motor wiring, the rotor and its magnet, and the primary rotor encoder as the location of the fault, because none of those swaps carried the abnormal behaviour with the part. Conversely, interchanging the secondary output-encoder PCB did move the behaviour between joints, localising the fault to a physical hardware difference at that board. The subsequent electrical reverse-engineering then narrowed what that difference could be: the board is a datasheet-normal ExtSSI iC-MU node whose stored configuration decodes as internally consistent (CRC-valid), which removes configuration corruption as an explanation and points to a physical device-level difference rather than a programming one. The overarching limitation is that the hardware work stops, by design, at “the fault is a physical hardware difference”; the confirmation of that fault at the signal level, the comparative read establishing the two encoders’ configurations byte-identical, and the eventual repair are reported in [Ming, SS-TBD].

The most concrete hardware limitation was the bench read that could not be completed. Reading the iC-MU’s raw registers in situ was defeated by two independent obstacles: the absence of a suitable RS-485/RS-422 interface (a full-duplex four-wire transceiver, or two RS-485 transceivers), and the driver STM32 remaining unpowered because its 3.3 V LDO rail did not come up under bench conditions. Each obstacle implies its own improvement: procuring the correct transceiver hardware to read the sensor directly, and working out the LDO’s enable condition so the driver can be powered and commanded to poll the encoder on the bench. A further, per-joint limitation is the residual mechanical offset left after the encoder swap: correcting it by re-clocking the rotor housing is possible only in coarse 45° steps given the eight securing screws, so a finer calibration route would be needed to remove it cleanly. Finally, the epoxy coating over the controller board prevented direct probing of several test points, a practical constraint on any future in-situ electrical work.

# Conclusion

The hardware objectives of the project were met. Spot’s power system was rebuilt from two dead modules into two modules that charge to a full, balanced state, by replacing the degraded cells with OEM-spec Samsung INR18650-30Q cells on the original BMS and, critically, by driving the rebuild’s reliability through spot-weld quality and a second-generation bracket design. In parallel, the faulty hip-X actuator, a previously undocumented piece of hardware, was characterised from the outside in: its 50:1 harmonic-drive, dual-encoder construction was established by teardown; a disciplined sequence of component swaps localised the fault to a physical difference at the secondary output-encoder board; and reverse-engineering that board established its ExtSSI iC-MU design, pinout, direct-to-STM32 routing and CRC-valid configuration, narrowing the fault to a physical device difference rather than a programming or wiring one.

The skills developed span the rebuild and characterisation of lithium battery systems and their BMS, the mechanical teardown and reassembly of a precision harmonic-drive actuator, and the electrical reverse-engineering of a Hall-encoder PCB, including datasheet-level EEPROM decoding and pinout tracing. The principal thread of future work follows directly from the one hardware task that could not be completed: instrumenting the iC-MU sensor with the correct RS-485/RS-422 interface (or a powered, polled driver STM32) to read its raw output on the bench. Beyond that, a physics-based simulation of the actuator and its encoder signal chain is proposed as a means of exploring the fault without repeated teardown; it is noted here strictly as planned work, no such simulation having been built within this project. The signal-layer confirmation of the fault and the encoder repair that returned the joint toward service are reported in the companion report [Ming, SS-TBD].

# References

[1] Boston Dynamics, “Spot Battery Safety Data Sheets (SDS).” Available: https://support.bostondynamics.com/s/article/Spot-Battery-Safety-Data-Sheets-SDS-49922

[2] Falcon PEV, “18650 Samsung 30Q 3000 mAh Battery Cell.” Available: https://www.falconpev.com.sg/products/18650-samsung-30q-3000mah

[3] “Comprehensive Teardown Report of the Quadruped Robot Spot,” r/IndiaTech, Reddit. Available: https://www.reddit.com/r/IndiaTech/comments/1nwsba2/comprehensive_teardown_report_of_the_quadruped/

[4] LHS Materials, “Thermal Regulation.” Available: https://www.lhsmaterials.com/thermal-regulation

[5] iC-Haus, “iC-MU Position Encoder — Pole Width 1.28 mm,” datasheet. Available: https://ave-nl.com/wp-content/uploads/2021/12/MU_datasheet_F2en_AVE.pdf

[6] Analog Devices, “LTC2863 — RS-485/RS-422 Transceiver,” datasheet. Available: https://www.analog.com/media/en/technical-documentation/data-sheets/2862345fc.pdf

[7] Analog Devices, “LT3029 — Dual Low-Dropout Linear Regulator,” datasheet. Available: https://www.analog.com/media/en/technical-documentation/data-sheets/3029fb.pdf

[8] Boston Dynamics, “Spot Software Updates.” Available: https://support.bostondynamics.com/s/article/Spot-Software-Updates-70795

# Appendices

## Appendix A — Project timeline (Gantt)

The hardware work reported here ran from mid-May to mid-July 2026 across the phases below (dates from the project’s session logs). Diagnostic-interpretation and repair sessions on the signal layer are the companion report’s and are omitted.

| Phase | Sessions (2026) | Dates |
| --- | --- | --- |
| Battery diagnosis & decision to rebuild | Inspection → restoration attempt → disassembly & sourcing | 13, 20, 21 May |
| Pack #1 design | Pack-design research; spacer CAD | 25 May |
| Pack #1 assembly & welding | Assembly sessions 1-4 | 28, 29 May; 2, 4 Jun |
| Pack #1 test, fault-find & rework | Cell-group diagnosis + DB25 bypass; weld rework & verify | 5, 11 Jun |
| Spot teardown / leg extraction | Teardown; pack #2 disassembly | 15 Jun |
| Pack #2 (V2) | V2 design; assembly; SoC-cable repair & full charge | 16, 17, 18 Jun |
| Actuator swaps | Driver & cable swaps | 18, 19 Jun |
| Motor disassembly | Secondary encoder identified | 25 Jun |
| Output-encoder swap | PCB-only swap; ROM/offset measurement | 30 Jun |
| Encoder electrical RE | EEPROM decode; routing; bench read attempt | Jun-16 Jul |

## Appendix B — iC-MU EEPROM configuration register map

Full annotated decode of the external configuration EEPROM against the iC-MU datasheet [5]. Raw dump (hex), addresses 0x00-0x3F:

00000000 00 00 00 00 00 88 00 00 00 00 00 07 00 00 14 05
00000010 00 4e 20 ff 0f 13 10 02 00 00 00 00 00 00 00 00
00000020 00 52 a8 ff ff ff ff ff ff ff ff ff ff ff ff ff
00000030 00 00 00 00 00 00 00 00 4d 55 07 00 00 00 69 43

| Addr | Raw | Register / fields | Decoded setting |
| --- | --- | --- | --- |
| 0x00 | 00 | GC_M, GF_M | Master coarse/fine gain 0 (ignored, amplitude control enabled). |
| 0x01 | 00 | GX_M | Master cosine gain adjust = 1.000. |
| 0x02 | 00 | VOSS_M | Master sine offset = 0 mV. |
| 0x03 | 00 | VOSC_M | Master cosine offset = 0 mV. |
| 0x04 | 00 | PH_M | Master phase adjust = 0°. |
| 0x05 | 88 | ENAC=1, CIBM=8 | Amplitude control active; bias-current trim nominal 0%. |
| 0x06 | 00 | GC_N, GF_N | Nonius coarse/fine gain 0 (ignored). |
| 0x07 | 00 | GX_N | Nonius cosine gain adjust = 1.000. |
| 0x08 | 00 | VOSS_N | Nonius sine offset = 0 mV. |
| 0x09 | 00 | VOSC_N | Nonius cosine offset = 0 mV. |
| 0x0A | 00 | PH_N | Nonius phase adjust = 0°. |
| 0x0B | 07 | MODEA=7, MODEB=0 | Port A = ExtSSI (NPRES/MA/SLI/SLO); Port B = ABZ (A/B/Z/NER). |
| 0x0C | 00 | CFGEW | All error/warning messages enabled. |
| 0x0D | 00 | ACC_STAT, NCHK_CRC, NCHK_NON, ACRM_RES, EMTD | Actual (non-accumulated) status; cyclic CRC + nonius verification active; no auto-reset; min error-display time 0 ms. |
| 0x0E | 14 | LIN=1, FILT=4 | Linear/radial scanning; FILT4 = 39 dB suppression, 14-bit interpolation. |
| 0x0F | 05 | MPC=5 | 32 master / 31 nonius periods; ≤19-bit absolute resolution. |
| 0x10 | 00 | *_MT | No external multiturn data; MT chain/verification disabled. |
| 0x11 | 4E | OUT_ZERO=2, OUT_MSB=14 | Serial output inserts 2 zero bits; output MSB = user-data bit 27. |
| 0x12 | 20 | MODE_ST=2, … | Binary output, no SSI ring; raw master/nonius track data; LSB = bit 0. |
| 0x13-0x14 | FF 0F | RESABZ=0x0FFF | ABZ / FlexCount resolution = (0x0FFF + 1) × 4 = 16 384 edges. |
| 0x15 | 13 | SS_AB=1, FRQAB=3, … | AB step 1; startup counting visible; AB limit ≈ 781.25 kHz (320 ns edge distance). |
| 0x16 | 10 | LENZ, CHYS_AB=1, PP60UVW, INV_* | Z index = 90°; AB hysteresis 0.175°; UVW 120°; A/B/Z not inverted. |
| 0x17 | 02 | RPL=0, PPUVW=2 | Config mode, no restriction; UVW commutation = 1 pole pair. |
| 0x18 | 00 | TEST | Normal mode. |
| 0x19-0x20 | 00 | SPO_BASE, SPO_0…14 | Nonius track-offset slopes all zero. |
| 0x21-0x22 | 52 A8 | CRC16 | 0x52A8, matches CRC over config ranges 0x00-0x20, 0x30-0x3F. |
| 0x23-0x27 | FF | OFF_ABZ | Absolute-output offset block reads erased (unused). |
| 0x28-0x29 | FF FF | OFF_UVW | UVW offset reads erased. |
| 0x2A-0x2E | FF | PRES_POS | Preset position reads erased. |
| 0x2F | FF | CRC8 | Does not match posted offset/preset bytes (block unused). |
| 0x30 | 00 | PA0_CONF | Falling edge on NPRES = NO_FUNCTION. |
| 0x31 | 00 | EDSBANK | No EDS. |
| 0x32-0x33 | 00 00 | PROFILE_ID | 0x0000. |
| 0x34-0x37 | 00 | SERIAL | 0x00000000. |
| 0x38-0x3D | 4D 55 07 00 00 00 | DEV_ID | ASCII “MU”, then 0x07 00 00 00. |
| 0x3E-0x3F | 69 43 | MFG_ID | 0x6943 = ASCII “iC” (iC-Haus). |

## Appendix C — Encoder-board pinout

See Figure 28 and Table 6 for the ExtSSI wiring, and Figure 29 for the connector-flex test pads (the transceiver’s A/B/Y/Z differential pairs plus +5 V and ground) used for the bench read attempt of Section 7.

Figure 29 — The connector-flex test pads, mapping to the transceiver’s A/B/Y/Z outputs and the encoder supply rails.

## Appendix D — DB25 payload-port cover pin shorts

The four sets of pins shorted inside the OEM payload-port cover, reverse-engineered by continuity probing and replicated on the open port to bypass the cover-detection lockout (Section 7).

Figure 33 — The DB25 payload-port cover pin shorts. Replicating these four shorts on the open port clears the “uncovered payload port” motor-power lockout.

## Appendix E — Battery CAD iterations

The V1 cell spacer (Figure 8) reproduced the irregular OEM cell positions in PC-CF. The V2 brackets (Figure 18) added bracing, OEM-derived spacer positions, ABS-GF construction and captured-nut BMS mounting, and are compared against the cleaner pack #2 weld result in Figure 19.